Geophysical Time Series Analysis & Digital Signal Processing:

Mid-term exam, part 1, March 2016

| Name: |  |
| --- | --- |
| Date: |  |

# Week 1

  - How would you combine the log2 and ceil functions to compute the minimum number of bits needed to hold an, arbitrary positive integer, x? (requires only 1 line of code)

  - What is the minimum number of bits needed to represent the following integers?

    - 33

    - 999

    - 123456789

  - The following numbers are given in MATLAB’s version of scientific notation. Write them out as normal numbers (e.g. 1.0e1 = 1.0 x 101 = 10). Bonus points if you can name the physical constants they represent:

    - 2.99792458e8 	(units m/s)

    - 1/1.3703599908e2   (dimensionless)

    - 6.67408e-11 (m3 kg-1 s-2)

  - What special mathematical constants are represented by the following numbers?

    - 0.3141592653589793e1

    - 1/(1.61803398875e0)+1

    - 1/(3.67879441171442e-1)

  - Describe in a few words the following special MATLAB constants:

    - Inf

    - NaN

    - j

    - eps

    - realmax

    - realmin

  - For each of the following supply the 1-line of MATLAB code needed:

    - Create a row vector x of 1000 uniformly distributed numbers between -3 and 3.

    - Create a column vector y of 1000 uniformly distributed integers from 1 to 100.

    - Create a 500x1 matrix, r, with normally distributed values, with a mean of 2.0 and a standard deviation of 3.0.

    - Create a 10x10 identity matrix, M.

    - Create a 1000x1 vector, z, of zeros. (Useful for initializing a vector/matrix).

    - Create a vector, s, of the sequence from -pi to +pi in steps of pi/100.

  - To solve a quadratic equation ax2+bx+c=0 you probably learned this formula in school: . The +/- means there are two solutions. Write either solution using just 1-line of MATLAB code. (This is to test your use of mathematical operator symbols and parenthesis).

  - Assuming 8-bit two’s complement, what decimal numbers are represented by the following binary sequences?

    - 01010101

    - 10101010

    - 11111111

  - Computers can only add. So it does the equivalent of 55-99 = 55 + (-99). What is:

    - 55 in 8-bit binary?

    - 99 in 8-bit binary?

    - The two’s complement of b?

    - The sum of the results from parts a and c (still in 8-bit binary)?

    - Convert d back into decimal.

  - What amplitude ratio is represented by the following decibel differences?

    - 20 dB?

    - 6 dB?

    - 120 dB?

  - In computing, what are overflow and underflow?

  - John writes a MATLAB code to output numbers from 0.1 to 1.0 in steps of 0.1.

x=0.1;

while x~=1

disp(x)

x=x+0.1;

end

    - How many times will the while loop execute?

    - Explain your answer to part a.

    - How would you fix this code by just changing 1 character?

    - What other type of loop would be better for this task?

# Week 2

  - What are three properties that make a system linear?

  - What is meant by the term “sinusoidal fidelity”?

  - Harmonic (sinusoidal) volcanic tremor causes a maximum ground motion of 1 cm/s. The largest ground motion the digitizer system can faithfully represent is 1 mm/s. Is this system linear?

  - Two linear systems A and B are combined in series. That is input x goes into A, results in output y which goes into B and comes out as z. What is the result is x goes into B first and then this output w goes into A?

  - What two ways can signals be combined to preserve linearity?

  - What is decomposition (in DSP)? Why is it so useful?

  - What are 4 common decompositions?

  - For a complex number z = x + jy, write formulae for the magnitude and the phase of z.

  - What set of functions can be represented by complex exponentials?

  - Write the difference equations for the following signals, x[n], in terms of unit step function, u[n], scaling, shifting, flipping and summing as needed:

    - x[n] = unit step scaled by -2 & starting at n = -4

    - x[n] = unit boxcar from n=-3 to n=5

    - x[n] = δ[n]

# Week 3

3.1 What mathematical operations are represented by the following difference equations:

- y[n] = x[n] – x[n-1]

- y[n] = x[n] + y[n+1]

3.2 A system H has an impulse response that is symmetric about time=0. A sine wave of frequency 2 Hz is delivered to system H. What is the phase delay? What is the phase delay for a 10 Hz sine wave?

3.3 What are 3 properties of convolution?

3.4 Convolve the following sequences by hand and show your working. x=[4 5 6], h=[-1 3].

3.5 Signal x is 1000 terms long. System h has an impulse response (filter kernel) that is 100 terms long. How long is the convolution x*h?

3.6 On the plot below, correctly label the unit circle, the pole, the zero and the Nyquist frequency. Also identify the region of z-values that correspond to (A) oscillating exponentials that decay (B) oscillations that neither grow nor decay (i.e. sinusoids) and (C) oscillating exponentials that grow.

3.7 The following diagram represents an 8-point Discrete Fourier Transform. Suppose instead a 16-point DFT were used. How would it look? Label all the points with the correct complex exponential.

3.8 Mathematically, what transform is the (continuous-time) Fourier Transform a special case of? What transform is the Discrete Fourier Transform a special case of?

3.9 What is the FFT and why is it important? (One or two sentences is enough).

3.10 Plot x[n] = 5(sin(0.39n)+sin(0.40n)), 0  n  1000. Comment on the waveform shape. What is this phenomenon called? How do musicians use it?

3.11 Plot a histogram (use 100 bins) of the signal y[n]=x[n] + x[n-1], where x is a random sequence whose samples are independent and uniformly distributed over [0,1] interval. Comment on the shape of the histogram and explain it. Compare it to throwing 2 dice.

3.12 Create 4 random sequences x1[n], x2[n], x3[n], x4[n]. Each should have 100,000 samples of uniformly distribution over interval [-0.5, 0.5]. Sum these sequences, i.e. y[n] = x1[n] + x2[n] + x3[n] + x4[n]. Plot a histogram (use 100 bins) of the signal y[n]. What shape is the histogram?

3.13 Repeat 3.12 but this time each of the sequences (x1, x2, x3, x4) should sample a normal distribution (mean 0, variance 1). What shape is the histogram of y[n] this time?

3.14 Consider a sequence, y[n], which is the linear combination of several random sequences – some of them uniformly distributed, others normally distributed. What shape do you expect the histogram of y[n] to have?

3.15 Derive the result that the sequence y[n]=sin(2fn) is only periodic if the normalized frequency, f, is a rational number (i.e. the ratio of two integers).

3.16 Which of the following sequences, y[n], are periodic? (hint: you should not need to plot them!)

- y[n] = cos(0.2n)+4

- y[n] = sin(0.2n)+4

- y[n] = cos(0.2n) + cos(0.2n)

- y[n]=e-jwn where w=0.2

- y[n]=e-jwn where w=0.2

- y[n]=e-wn where w=0.2

# Week 4

4.1 An analog signal is dominated by a 60 Hz. If the signal is digitized at a sampling rate of 100 Hz, and then the amplitude spectrum is taken, describe how it will look.

4.2 A digital signal has a broad spectrum from 1-40 Hz and a sampling rate of 100 Hz. It is then downsampled to a sampling rate of 50 Hz. Describe what processing should be done before downsampling.

4.3 You hope to find two peaks in the spectrum of a gravimeter record. Theory predicts they will have periods of 6.2 hours and 6.6 hours with good signal to noise ratio.

- For how long must you record in order to resolve two distinct peaks?

- What is a suitable sampling frequency, assuming no significant energy arrives with period of 6 hours?

- How many values will your time series contain?

4.4 Use the deconv function in MATLAB to deconvolve the filter h = (1,2,3,1,2) from the time series y=(1, 5, 14, 27, 34, 32, 17, 14). Write the answer here:

4.5 Write the z-transform of y in the previous question. Determine the zeros (roots) of this polynomial and plot them on the z-plane. Plot the impulse response and the frequency response. (Hint: use the MATLAB functions roots, zplane, impz and freqz).